DSG paintball marker instructions

WARNINGS:

- SAFE USAGE AND OPERATION OF THIS DEVICE IS ESSENTIAL TO MAINTAINING A SAFE PLAYING ENVIRONMENT.

- THIS DEVICE IS NOT A TOY.

- ALWAYS WEAR APPROVED SAFETY GOGGLES WHEN USING THIS DEVICE.

- ADULT SUPERVISION IS RECOMMENDED AT ALL TIMES.

- DO NOT LEAVE THIS DEVICE UNNATENDED WHILE PRESSURIZED.

- ALWAYS USE AN APPROVED BARREL BLOCKING DEVICE WHEN NOT PLAYING.

- CAUTION: This is not a toy and should be used only under adult supervision. Misuse or careless use may cause serious injury. The user of this device and all others nearby must wear paintball approved eye and face protection.

- This marker is powered by CO2 gas under high pressure. Caution should be taken with both the marker and the gas source.

- Always discharge your marker by pulling the trigger before leaving the field of play. Never leave the marker in a cocked and ready position when not playing.  Always replace your barrel blocking device before leaving the field.

- Always chronograph your marker and adjust your velocity to safe field speeds before each day of play.  Never use this marker at speeds above 300 feet per second as serious injuries may occur.

- Never leave your marker in direct sunlight, this can dramatically increase its temperature and directly affect the pressure of the CO2 inside.  CO2 pressure spikes can increase velocities to unsafe levels.

- Always point your lever changer in a safe direction when discharging a 12 gram CO2 cylinder.  If it is not empty it could be propelled dangerously by remaining gasses.

- Always use caution when loading and discharging CO2 cylinders, they contain liquefied gas at high pressures.  When released, this liquid and or gas can cause severe burns and frostbite.

USING YOUR MARKER

  - Place your barrel blocking device on the marker.

  - Unload any existing CO2 cylinder in a safe manner.

  - Discard any paint that may be loaded in the marker from previous days play.

  - Pull the trigger to release the hammer.

  - Load fresh, quality paint into the feed tube of the marker and replace the spring tube.

- Always point your marker up and in a safe direction to load a fresh 12gram CO2 cartridge.  This will help prevent liquid CO2 from entering the valve.

  - Adjust the lever changer to get a solid latch and reduce the chance of leaks in the pierce pin area.  This can be adjusted with the small black knob on the lever changer.  It is best to keep empty 12 gram cylinders on hand from different brands to accomplish a proper adjustment.

  - Chronograph your marker to a safe field speed before discharging it at anyone or anything.  The velocity adjustment can be accomplished with the external velocity key (if so equipped), or with a hex key through the grip frame screw hole ( if not equipped with external velocity adjustment).  Clockwise will increase velocity, counterclockwise will decrease velocity.

  - Always check for proper installation and components for safe operation.

MAINTENANCE

- Always clean and lubricate your marker after each day of play.

- Use air tool oil to regularly lubricate the internal moving pieces of your marker.

- Do not over tighten fasteners as you may damage components.

- When using hex keys, always insert the long side into the fastener and tighten with the short side.  All the fasteners on this marker use imperial sizes.  Use of metric or other size hex keys will damage the fasteners and possible prevent their future removal.

- NEVER use a hex key on the bolt screw.  The bolt is made of soft plastic and will be damaged.  It may also cause the screw to jam in the bolt preventing its future removal.  Tighten it with finger pressure only, and check for proper installation regularly.

- Never interchange the different fasteners on this marker.  This may cause damage to the fasteners and components.

- Watch the assembly and disassembly videos available at:

- http://s325.photobucket.com/albums/k372/duke737/DSG/?action=view&current=Disassembly.flv

- http://s325.photobucket.com/albums/k372/duke737/DSG/?action=view&current=Reassembly.flv

- An assembly diagram is available at:

- http://www.mcarterbrown.com/forums/attachments/dealers-forum/11728d1255383649-second-dsg-sales-thread-1709023-dsg-assembly.zip

WARRANTY

There are no warranties, express or implied, of merchantability or fitness for any purpose except as follows.

This paintball marker is warranted to the original purchaser to be free from defects in material and workmanship for a period of ninety (90) days from the original date of purchase. This warranty is limited to replacement or repair of defective parts by Dart Engineering Inc.

This warranty does not apply to components or assemblies that have been modified, misused, damaged, poorly maintained, or worn through normal usage.

CONTACT INFORMATION

Dart Engineering Inc.

Unit 22- 6 Colby Court

Waterloo, ON

N2V 1Y9

Email: danm@dart-inc.com

Private message on www.mcarterbrown.com: http://www.mcarterbrown.com/forums/members/dukie.html